Cable Specifications

The bridge units are shipped with a short adapter cable that can be used to attach the SLH10/12 antennas directly to the bridge unit.

Adapter Cable Suplied with the Bridge:

To connect to the bridge:	RFS-2006 SMA Male Right Angle Crimp

with Modified Shell by RF Industries

To connect to the antenna:		RFT-1202-c TNC Male Reverse Threaded

as manufactured by Times

Cable type				LMR-195, maximum length 3 feet

(multiple manufacturers such as Belden,

Times, etc.)

If the antenna must be located further away from the bridge unit, then a new adapter cable and an extension cable must built.  Under no circumstances should the antennas be mounted more than 50 feet away from the bridge units as the loss of signal strength and increased noise are substantial.

Adapter Cable for use with an Extension Cable:

To connect to the bridge:	RFS-2006 SMA Male Right Angle Crimp

with Modified Shell by RF Industries

To connect to the 			RFN-1005-3C N Type Male Crimp

Extension Cable			as manufactured by RF Industries or similar

Cable type				LMR-195, maximum length 3 feet

(multiple manufacturers such as Belden,

Times, etc.)

Extension Cables:

To connect to the 			RFN-1028-S1 N Type Female Crimp

Adapter Cable				as manufactured by RF Industries or similar

To connect to the antenna:		TC-400-TM-RT TNC Male Reverse Threaded

as manufactured by Times

Cable type				LMR-400, maximum length 50 feet

(multiple manufacturers such as Belden,

Times, etc.)

Sources for Connectors and Cables

Many different manufacturers can supply the LMR400 and LMR195 cable as well as the Type N connectors.  At the time of this printing, the following sources are available for these products:

Electro-Comm Distributing

Denver, Colorado

800-525-0173

303-371-8182

Kimball Electronics

Salt Lake City, Utah

801-466-0569

RF Industries

San Diego, California

800-233-1728